# Prof. Raghuveer Parthasarathy

University of Oregon; Fall 2024

## Physics 410/510 Image Analysis:  Homework 5

Solutions

1 The Signal-to-Noise Ratio in Images. ... Does this matter?

No, it doesn’t matter – the SNR is independent of any rescaling of the pixel values.

I’ll just do the “math” version of the solution, which is the better thing to do anyway:

Measurements: x = {x1, x2, x3, ..., xN}

The mean:

<x> = (1/N) (x1+x2+x3+... xN)

Multiplying by A:

Ax = {Ax1, Ax2, Ax3, ..., AxN}

<Ax> = (1/N) (Ax1+Ax2+Ax3+... AxN)

= A (1/N) (x1+x2+x3+... xN) = A <x>

The variance is the mean of the squared difference of each point to the mean:

var(x) = < (x-<x>)2 >

= (1/N)  [ (x1-<x>)2 + (x2-<x>)2 + ... + (xN-<x>)2]

Multiplying by A:

var(Ax) = (1/N)  [ (Ax1-A<x>)2 + (Ax2-A<x>)2 + ... + (AxN-A<x>)2]

= A2 var(x), since we can factor out A2.

The standard deviation is the square root of the variance, so

std(Ax) = A std(x)

Therefore the SNR is A <x>/ [A std(x)], which clearly doesn’t depend on A.

2 Centroid warmup

(i) xc = 599.71

(ii) xc = about 445 to 450 (it’s random, of course)

[next time: do N = 7 rather than 11; do 40 to 40,000 photons for (b)]

3 Centroid localization.

(a) I calculated simulated images with a fine grid size of 2 nm, then pixelating and adding noise as in HW3. There are many ways to write the centroid calculation, but all will involve some multiplication of positions and intensities.

For example, you can make a grid of X and Y pixel positions:

Nx = im.shape[1]

Ny = im.shape[0]

x = np.arange(Nx) - (Nx-1)/2

y = np.arange(Ny) - (Ny-1)/2

xx, yy = np.meshgrid(x, y) and then the centroid of image k is

xc = np.sum(xx*im)/np.sum(im)

yc = np.sum(yy*im)/np.sum(im) here, the centroid position is calculated in pixels.

The RMS error is

RMSE = np.sqrt(np.mean((xc-x0)**2 + (yc-y0)**2))

where the mean is over all trials.

For M = 100 trials, this gives:

RMSE: 0.011 microns (0.11 pixels), and the following histogram of xc values:

(b) I made 100 images at each of 20 Nphoton values. Here’s the graph:

The solid line has slope -1/2, i.e. 1/sqrt(N), which we might expect. The data don’t really follow this, except maybe at large Nphoton. The error does decrease with increasing photon number, though!

(c) The method is the same as for part (a).

Here are the histograms:

(x0, y0) = (0.0, 0.0) px. We see that the error is roughly centered at 0.

(x0, y0) = (0.3, 0.0) px: We see that the error is *not* centered at 0. The estimated center is less than (left of) the true center!

(x0, y0) = (-0.3, 0.0) px. We see that the error is *not* centered at 0. The estimated center is greater than (right of) the true center!

[Maybe do different Nphotons also?] 100,000

(d) Your plot should look like this:

The error is a linear function of the true object position. The centroid is biased towards the image center, as we also saw in (c).

Also do this for a Poisson distributed background with mean = 100.